Nguyễn An Phúc – 21IT639

Ôn tập

Câu 1. In the CPU, what is the functionality of the control unit?

A. To decode program instruction

B. To perform logic operations

C. To store program instruction

C.To transfer data to primary storage

Câu 2. The internal components of the processor are connected by _______

A.   Processor intra-connectivity circuitry

B.   Processor bus

C.   Memory bus

D.   Rambus

Câu 3. The brain of the computer. This part does the calculation, moving and processing of information?

A. CPU

B. RAM

C. Motherboard

D. Hard Drive

Câu 4. What is the capacity of the ENIAC machine?

A.            2000 additions/sec

B.            3000 additions/sec

C.            4000 additions/sec

D.            5000 additions/sec

Câu 5. The time required to fetch an instruction from memory and execute it is called _____

A.            Fetch Cycle

B.            Execution Cycles

C.            Load Cycle

D.            Instruction Cycle

Câu 6. The basic structure of an instruction has ........

A.            Register and Opcode

B.            Decoder and Accumulator

C.            Sequence register and Decoder

D.            Opcode and Address

Câu 7. When ever the data is found in the cache memory it is called as _________

A.            HIT

B.            MISS

C.            FOUND

D.            ERROR

Câu 8.  Consider a machine with a byte addressable main memory of 2^16 bytes and block size of 16 bytes. Assume that a direct mapped cache consisting of 64 lines is used with this machine. How is a 16-bit memory address divided into tag number, line number, and byte number?

A.            7-5-4

B.            8-4-4

C.            6-6-4

D.            6-7-3

Câu 9. How many address bits are required to represent a 32 K memory[1] ?

A. 10 bits

B. 12 bits

C. 14 bits

D. 15 bits

Câu 10. A databased record has 4 fields. FirstName (text up to 10 characters), Surname (text up to 10 characters), DateOfBirth, NumberOfChildren. There are are 100 records. Text = 1 byte per character, Integer = 4 bytes, Real = 4 bytes, Date = 8 bytes, Boolean = 1 byte. How much storage in bytes would this take up? (Including overheads)

A. 3520

B. 352

C. 35200

D. 352000

Câu 11. Which statement is incorrect?

A. SSDs have lower power consumption than HDDs

B. SSDs have lower cost per bit than HDDs

C. SSDs have lower latency than HDDs

D. SSDs have longer lifespan than HDDs

Câu 12. What is the main advantage of using external memory?

A. It is faster than internal memory

B. It is more expensive than internal memory

C. It is more secure than internal memory

D. It has a larger storage capacity

Câu 13. In which IO mechanism, does the processor take the least control on I/O operation?

A. Programmed

B. Interrupt-driven

C. DMA

D. Programmed or Interrupt-driven

Câu 14. In DMA transfers, the required signals and addresses are given by the ______

A. Processor

B. Device drivers

C. DMA controllers

D. The program itself

Câu 15. For the following CPP program, what is the result on the screen when executing the program?

A. 217

B. 177

C. 945

D. 257

Câu 16. For the following CPP program, what is the result on the screen when executing the program?

A. -79

B. 177

C. 945

D. 217

Câu 17.  When we perform subtraction on -7 and -5 the answer in 2’s complement form is _____

A.                                                 1111

B.                                                 1110

C.                                                 1010

D.                                                 0010

Câu 18.  In which addressing mode is the operand value directly specified in the instruction?

A.                                                 Immediate

B.                                                 Relative

C.                                                 Direct

D.                                                 Definiti

Câu 19. If a processor clock is rated as 1250 million cycles per second, then its clock period is _____

A. 1.9 * 10^-10 sec

B. 1.6 * 10^-9 sec

C. 1.25 * 10^-10 sec

D. 8 * 10^-10 sec

Câu 20.  Assembly language programs are written using

A.   Hex code

B.   Mnemonics

C.   ASCII code

D.   Binary code

Câu 21.  __________ converts the programs written in assembly language into machine instructions.

A.                                                 Machine compiler

B.                                                 Interpreter

C.                                                 Assembler

D.                                                 Converter

Câu 22. Given the following assembly language

After executing the above program, the AH register has the value (decimal) is:

A. 10

B. 11

C. 12

D. 13