BÀI TẬP: THỰC HÀNH CẤU HÌNH PORT-SECURITY

1. Sơ đồ

2. Lệnh cấu hình

AnPhuc#show mac address-table

Mac Address Table

-------------------------------------------

Vlan Mac Address Type Ports

---- ----------- -------- -----

1 0040.0b82.e46a STATIC Fa0/2

1 00d0.9727.57b3 STATIC Fa0/1

AnPhuc#sh

AnPhuc#show por

AnPhuc#show port-security ad

AnPhuc#show port-security address

Secure Mac Address Table

-----------------------------------------------------------------------------

Vlan Mac Address Type Ports Remaining Age

(mins)

---- ----------- ---- ----- -------------

1 00D0.9727.57B3 SecureSticky Fa0/1 -

1 0040.0B82.E46A SecureSticky Fa0/2 -

-----------------------------------------------------------------------------

Total Addresses in System (excluding one mac per port) : 0

Max Addresses limit in System (excluding one mac per port) : 1024

AnPhuc#show port-security interface fa0/1

Port Security : Enabled

Port Status : Secure-up

Violation Mode : Restrict

Aging Time : 0 mins

Aging Type : Absolute

SecureStatic Address Aging : Disabled

Maximum MAC Addresses : 1

Total MAC Addresses : 1

Configured MAC Addresses : 0

Sticky MAC Addresses : 1

Last Source Address:Vlan : 0060.5C70.E86E:1

Security Violation Count : 36

AnPhuc#show port-security interface fa0/2

Port Security : Enabled

Port Status : Secure-up

Violation Mode : Restrict

Aging Time : 0 mins

Aging Type : Absolute

SecureStatic Address Aging : Disabled

Maximum MAC Addresses : 1

Total MAC Addresses : 1

Configured MAC Addresses : 0

Sticky MAC Addresses : 1

Last Source Address:Vlan : 0040.0B82.E46A:1

Security Violation Count : 0

AnPhuc#

3. Kết quả : PC1 ping được, PC0 không ping được

4. Lệnh show running

AnPhuc#show running-config

Building configuration...

Current configuration : 1704 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname AnPhuc

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

switchport port-security mac-address sticky 00D0.9727.57B3

!

interface FastEthernet0/2

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security violation restrict

switchport port-security mac-address sticky 0040.0B82.E46A

!

interface FastEthernet0/3

shutdown

!

interface FastEthernet0/4

shutdown

!

interface FastEthernet0/5

shutdown

!

interface FastEthernet0/6

shutdown

!

interface FastEthernet0/7

shutdown

!

interface FastEthernet0/8

shutdown

!

interface FastEthernet0/9

shutdown

!

interface FastEthernet0/10

shutdown

!

interface FastEthernet0/11

shutdown

!

interface FastEthernet0/12

shutdown

!

interface FastEthernet0/13

shutdown

!

interface FastEthernet0/14

shutdown

!

interface FastEthernet0/15

shutdown

!

interface FastEthernet0/16

shutdown

!

interface FastEthernet0/17

shutdown

!

interface FastEthernet0/18

shutdown

!

interface FastEthernet0/19

shutdown

!

interface FastEthernet0/20

shutdown

!

interface FastEthernet0/21

shutdown

!

interface FastEthernet0/22

shutdown

!

interface FastEthernet0/23

shutdown

!

interface FastEthernet0/24

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end

AnPhuc#